## U-Net vs. YOLOv8: A Comprehensive Comparison Focused on Speed and Precision

### 1. Introduction

In deep learning–based computer vision projects, model selection is one of the most critical factors determining overall system success. While U-Net is the standard architecture in domains such as medical imaging, where pixel-level precision is crucial, YOLOv8 represents a state-of-the-art architecture that combines speed and accuracy for real-time applications.
This section presents a comprehensive comparison of both models, focusing on their advantages, limitations, and specific optimization modules that improve inference speed and output sharpness.

### 2. U-Net Model: Pixel-Level Precision

U-Net is a semantic segmentation architecture that performs pixel-by-pixel classification, making it particularly suitable for scenarios where object boundaries must be clearly and accurately defined.

#### 2.1. Advantages

- High Pixel Accuracy:
By analyzing every pixel in the image, U-Net extracts object boundaries (contours) with very high precision.

- Strong Performance with Limited Data:
Through data augmentation techniques, U-Net can achieve stable performance even with relatively small datasets.

#### 2.2. How Can U-Net Be Accelerated? (Key Modules and Techniques)

Standard U-Net architectures are computationally heavy; however, several techniques enable deployment on embedded systems such as Raspberry Pi and Jetson platforms:

- Backbone Replacement (Mobile U-Net):
Replacing the encoder part of U-Net with lightweight backbones such as MobileNetV3 or EfficientNet can reduce the number of parameters by up to 90%, resulting in significant speed improvements.

- TensorFlow Lite (TFLite) and Quantization:

  - Converting the model to .tflite format reduces model size.

  - INT8 Quantization: Reducing weights from 32-bit floating point to 8-bit integers typically increases inference speed by 2–3×, with minimal impact on accuracy.

- Input Resolution Optimization:
Reducing input image resolution (e.g., from 512×512 to 256×256) decreases computational load quadratically, leading to substantial performance gains in U-Net architectures.

### 3. YOLOv8 Model: Speed and Versatility

Although YOLOv8 is primarily known for object detection, the YOLOv8-Seg variant introduces instance segmentation capabilities through a dedicated segmentation head.

#### 3.1. Advantages

- Real-Time Performance (State of the Art):
YOLOv8 currently represents the industry standard for speed–accuracy trade-offs.

- Single-Stage Architecture:
The model processes the image in a single forward pass, simultaneously detecting objects and generating segmentation masks.

#### 3.2. How Can YOLOv8 Produce Sharper Results? (Key Modules and Techniques)

YOLOv8 may occasionally miss small objects or produce less smooth mask boundaries compared to U-Net. The following techniques mitigate these limitations:

- SAHI (Slicing Aided Hyper Inference):

  - Description: An open-source method that divides large images into smaller slices before inference.

  - Effect: Significantly improves small-object detection and mask boundary sharpness. Fully compatible with YOLOv8.

- YOLOv8-Seg (Segmentation Head):
Using -seg models (e.g., yolov8n-seg.pt) enables direct mask generation instead of bounding boxes, improving object shape accuracy.

- Test-Time Augmentation (TTA):
Processes the same image at multiple scales and orientations during inference, then merges the results. This slightly reduces speed but improves accuracy and edge sharpness.

### Note: SAHI (Slicing Aided Hyper Inference)

#### Advantages (+)

- Superior Small-Object Detection (Key Strength):
Standard YOLO models resize the input image, causing distant or small objects (e.g., medicine boxes, keys, coins) to be lost. SAHI effectively “zooms in” by slicing the image, enabling highly reliable detection of small objects. This is particularly valuable for visually impaired users when identifying small items on surfaces.

- No Retraining Required:
Existing YOLOv8 models or TensorRT engines can be used directly. SAHI is a method, not a new model, eliminating the need for additional training.

- Preserves Camera Resolution:
While YOLO typically downsamples high-resolution inputs (e.g., 1080p → 640p), SAHI processes full-resolution images by slicing, preserving fine visual details.

#### Disadvantages (–)

- Significant FPS Reduction (Critical for Jetson Nano):
Instead of processing one frame once, SAHI may require 4–9 inferences per frame, reducing performance from ~30 FPS to 3–4 FPS.

- Increased Latency:
The pipeline (slice → infer → merge) introduces noticeable delay. For a walking user who requires immediate feedback, this latency may pose safety risks.

- Higher CPU/GPU Load and Thermal Stress:
SAHI keeps the GPU at high utilization for longer durations, increasing heat generation and accelerating battery drain—particularly problematic on resource-constrained platforms like Jetson Nano.